| Client: | CTICM/ARCELORMITTAL |  |
| --- | --- | --- |
| Order reference: | Order N° 45584318 on 23/12/22 and contract AM/CTICM on 12/01/23 |  |
| Project Index: Product Index: | DRV/DAL-22-003/002-A DRV/PMX/MV/001-A |  |
| ABC-PMX-II Software Composite beam design validation manual |  |  |

Membre du Comité de Coordination des Centres de Recherche en mécanique / Siret : 775 728 785 00046 Code Naf : 7219 Z

| Revision | Date | Authors | Subject |
| --- | --- | --- | --- |
| A | 26/01/2024 | G. DELACOURT | ABC-PMX-II Software – Validation Manual |
| B |  |  |  |
| C |  |  |  |
| D |  |  |  |

Content

1. Introduction	2

2. References	2

2.1. First generation of Eurocodes	2

2.2. Drafts for the second generation of Eurocodes	2

2.3. Scientific references	2

3. Principles of validation	3

4. Tests cases	6

4.1. Test case 1: Simply supported composite beam (RCM 2018-2)	6

4.1.1. Input data	Erreur ! Signet non défini.

4.1.2. Results	8

4.2. Test case 2: Simply supported composite beam with partial encasement during its construction phase (RCM 2023-3)	14

4.2.1. Input data	Erreur ! Signet non défini.

4.2.2. Results	17

4.3. Test case 3: Simply supported composite beam with partial encasement during its final phase (RCM 2023-4)	20

4.3.1. Input data	Erreur ! Signet non défini.

4.3.2. Results	21

4.4. Test case 4: Simply supported steel beam laterally restrained during its construction phase (RCM 2022-2)	12

4.4.1. Input data	Erreur ! Signet non défini.

4.4.2. Results	13

4.5. Test case 5: Simply steel beam with cantilevers	26

4.5.1. Input data	Erreur ! Signet non défini.

4.5.2. Results	28

4.6. Test case 6: Simply supported composite beam with welded steel section	31

4.6.1. Input data	Erreur ! Signet non défini.

4.6.2. Results	50

# Introduction

ABC-PMX software aims at the calculation of composite beams according to EN 1994-1-1. Its development is co-funded by ArcelorMittal and CTICM. This documents constitutes the Validation Manual.

# References

## First generation of Eurocodes

- EN 1990 – Basic of structural design. AFNOR. March 2003.

- EN 1991-1-2: Eurocode 1: Actions on structures Part 1-2: General actions — Actions on structures exposed to fire

- EN 1992-1-1 – Design of concrete structures. Part 1-1: General rules and rules for buildings. AFNOR. October 2005.

- EN 1993-1-1 – Design of steel structures. Part 1-1: General rules and rules for buildings. AFNOR. October 2005.

- EN 1993-1-2 – Design of steel structures. Part 1-2: General rules - Structural fire design

- EN 1993-1-5 – Design of steel structures. Part 1-5: Plated structural elements. AFNOR. March 2007.

- EN 1994-1-1 – Design of composite steel and concrete structures. Part 1-1: General rules and rules for buildings. AFNOR. June 2005.

- EN 1994-1-2 – Design of composite steel and concrete structures. Part 1-2: General rules - Structural fire design

## Drafts for the second generation of Eurocodes

- prEN 1992-1-1 – Design of concrete structures. Part 1-1: General rules and rules for buildings. Document SC2N2034

- FprEN 1993-1-1 – Design of steel structures. Part 1-1: General rules and rules for buildings. Document SC3 N3504 – 26/11/21

- FprEN 1993-1-5 – Design of steel structures. Part 1-5: Plated structural elements. Document SC3 N3728 – 26/11/21

- prEN 1994-1-1 – Design of composite steel and concrete structures. Part 1-1: General rules and rules for buildings. CEN/TC250/SC4 document N 2394. November 2021 - Draft.

## Scientific references

- Design of floor structures for human induced vibrations. M. Feldmann, Ch. Heinemeyer, Ch. Butz, E. Caetano, A. Cunha, F. Galanti, A. Goldack, O. Hechler, S. Hicks, A. Keil, M. Lukic, R. Obiala, M. Schlaich, G. Sedlaceck, A. Smith, P. Waarts – Background document in support to the implementation, harmonization and further development in the Eurocodes – Joint Report – 2009 – JRC Scientific and Technical Reports

- European Recommandations for the Application of Metal Sheeting acting as a Diaphragm – Stresses Skin Design – ECCS European Convention for Constructional Steelwork – Report no 88 – 1995

- MARTIN P-O., RODIER A. - Design of the web-to-flange welds of a steel and concrete composite beam with a welded steel section – Revue Construction Métallique 1/2023

- MARTIN P-O., NGUYEN T-M. - Resistance to lateral torsional buckling of a secondary beam in construction phase – Restraint by the steel deck – Revue Construction Métallique 3/2022

- Déversement des poutres en acier – Collection CTICM (in French)

- Guide de conception pour les planchers minces à poutres incorporées - ECCS European Convention for Constructional Steelwork – Report no 83 – 1996

- COSFB Software – Technical specifications – 15/09/2020 – ArcelorMittal

- Nomogrammes – Méthode graphique pour déterminer la résistance au feu des structures en acier, selon l’EN 1993-1-2 :2005 – Centre Information Acier -

# Principles of validation

This report includes test cases for all configurations. For each of them, the following are specified:

- the input data of the test case ;

- software output data ;

- the results obtained by the reference ;

- the comparison between the software output results and those obtained by the reference.

The input data is strictly identical between the software and the reference.

The references values used for software validation come from Excel spreadsheets developed specifically at CTICM and scientific literature references.

The comparison of the results is based on the evaluation of the differences between the values provided by the software and those of the reference. This comparison will be limited to the main results (verification ratios). The intermediate results will therefore be neither detailed nor compared here.

The analysis of the results obtained and the deviations observed makes it possible to confirm that the ABC-PMX-II software produces values in accordance with the calculation methods applied.

# Scope grid

| Parameter | Test C01 | Test C01b | Test C02 | Test C03 | Test C04 |
| --- | --- | --- | --- | --- | --- |
| Type of section / beam | Composite beam | Composite beam | Composite beam | Composite beam / partial encasement | Composite beam / partial encasement |
| Type of profile | HR | HR | HR | HR | HR |
| Steel grade | S275 (EC3) | S275 (EC3) | S275 (EC3) | S355 M/ML | S355 EC3 |
| Construction phase | No | yes | Yes | Yes | No |
| Propping | unpropped | unpropped | unpropped | unpropped | unpropped |
| Cantilever spans | No | No | No | No | No |
| Type of slab | Composite | Composite | Composite | Composite | Composite |
| Steel deck label | Cofraplus 60 | Cofraplus 60 | Cofraplus 60 | Cofraplus 60 | Cofraplus 60 |
| Ribs orientations | Perpendicular | Perpendicular | Perpendicular | Perpendicular | Perpendicular |
| Restraint by the deck | - | - | Yes | No | No |
| Studs | 19-100 | 19-100 | - | - | 19-100 |
| Standard | Generation 1 | Generation 1 | Generation 1 | Generation 1 | Generation 1 |
| Type of design | Plastic | Plastic |  |  |  |

| Parameter | Test S01 | Test S02 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type of section / beam | Steel beam | Steel beam |  |  |  |
| Type of profile | HR | Welded |  |  |  |
| Steel grade | S355 M/ML | S355 JR |  |  |  |
| Construction phase | - | - |  |  |  |
| Propping | - | - |  |  |  |
| Cantilever spans | Left and right | No |  |  |  |
| Type of slab | Solid | Solid |  |  |  |
| Steel deck label | - | - |  |  |  |
| Ribs orientations | - | - |  |  |  |
| Restraint by the deck | - | - |  |  |  |
| Studs | - | - |  |  |  |
| Standard | Generation 1 | Generation 1 |  |  |  |
| Type of design | Plastic | Plastic |  |  |  |
| Shear buckling | No | Yes |  |  |  |

# Tests cases – Composite beams

## CASE C01: Single span composite beam

### Description

This test is based on the Construction Métallique review n°2018-2. It deals with a single span composite beam with a composite slab.

The reference values are taken from the paper. Because the calculation method of the software is more accurate, some values of the review may not match perfectly.

Figure 1 : Transversal view

### Parameters

Definition of the beam:

Definition of the slab:

Composite slab

Total depth:	td=120 mm

Perpendicular ribs

Steel profiled sheeting:	Cofraplus 60, thickness 0,75 mm

Prepunched sheeting

Definition of the profile:

Hot rolled profile

Profile:	IPE 450

Steel grade:	S275 EC3

Definition of the concrete:

Normal concrete

Concrete class:	C25/30

Density:	ρc=2500 kg/m3

Definition of the connection:

Studs:	19x100

One stud in each rib

Unpropped beam during construction

Definition of loads:

G1:	self-weight only

G2:	surface load 1,4 kN/m2

live load Q1:	surface load 2,5 kN/m2

Combination of loads:

ULS:	1,35 (G1+G2) + 1,5 Q1

SLS:	(G1+G2) + Q1

Design parameters:

Standard:	Eurocodes first generation

Design:	According to the class of cross-section

Relative humidity:	50 %

Shrinkage deformation:	325 x 10-6

Age of concrete:	t0=28 d for G2

t = 50 x 365 d

### Verification of results

| Case C01 Results: Steel profile | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Cross-sectional area : A = (cm2) | 98.8 | 98.8 |  |
| Shear area : Av = (cm2) | 50.9 | 50.8 | 0.20% |
| Second moment of area (y-y axis) : Iy = (cm4) | 33 740 | 33 743 | 0.01% |
| Second moment of area (z-z axis) : Iz = (cm4) | 1 676 | 1 672 | 0.24% |
| Elastic section modulus (y-y axis) : Wel,y = (cm3) | 1 500 | 1 500 |  |
| Elastic section modulus (z-z axis) : Wel,z = (cm3) | 176 | 176 |  |
| Plastic section modulus (y-y axis) : Wpl,y = (cm3) | 1 702 | 1 702 |  |
| Plastic section modulus (z-z axis) : Wpl,z = (cm3) | 276 | 272.8 | 1.17% |
| Torsional constant : It = (cm4) | 66.66 | 66.75 | 0.13% |
| Warping constant : Iw = (cm6) | 780 739 | 791 005 | 1.30% |
| Mass by length unit : mlin = (kg/ml) | 77.6 | 77.6 |  |
| Total mass : m = (kg) | 1086 | 1 086 |  |
| Total paint surface : S = (cm2) | 224 700 | 224 721 | 0.01% |
| Paint surface (without upper face) : S = (cm2) | 198 100 | 198 121 | 0.01% |
| Massiveness : M = (m-1) | 162 | 162.4 | 0.25% |
| Massiveness (without upper face) : M = (m-1) | 143 | 143.2 | 0.14% |
| Nominal yield strenght : fy = (MPa) | 275 | 275 |  |
| Young’s modulus : E = (MPa) | 210 000 | 210 000 |  |
| Density : ρ = (kg/m3) | 7 850 | 7 850 |  |
| Tensile resistance : Nt,Rd = (kN) | 2 717 | 2 718 | 0.04% |
| Shear resistance : Vpl,Rd = (kN) | 807 | 807 |  |
| Plastic moment resistant : Mpl,Rd = (kN.m) | 468 | 468 |  |
| Elastic moment resistant : Mel,Rd = (kN.m) | 412.5 | 412 | 0.12% |
| Section class for bending : | 1 | 1 |  |
| Section class for compression : | 4 | 4 |  |
| Shear buckling resistance : hw/tw = | 40.3 | 40.3 |  |
| Shear buckling resistance : 72ε/η = | 55.5 | 55.5 |  |
| Verification of web buckling required? | No | No |  |

| Case C01 Results: slab | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Notional size (EN 1992-1-1 Annex B) : h0= (mm) | 124 | 124 |  |
| Char. value of the compression strength : fck= (MPa) | 25 | 25 |  |
| Mean value of compression strength : fcm= (MPa) | 33 | 33 |  |
| Mean tensile strength : fctm= (MPa) | 2.56 | 2.56 |  |
| Mean secant modulus of elasticity : Ecm= (MPa) | 31 476 | 31 476 |  |
| Sec. moment of area per unit of width : Ipu= (cm4/m) | 44.37 | 44.37 |  |
| Shear resistance of the connector : PRd= (kN) | 52.9 | 52.9 |  |
| Reduction factor : kt = | 0.71 | 0.71 |  |

| Case C01 Results: Section properties at mid-span | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Effective width of the slab : beff= (m) | 3,0 | 3,0 |  |
| Modular ratio : n(G2)= | 27.5 | 27.5 |  |
| Modular ratio : n(Q1)= | 6.67 | 6.67 |  |
| Modular ratio : nSHC= | 26.1 | 26.1 |  |
| Second moment of area (y-y axis) : Iel,y(n=6.67) (cm4) | 106 570 | 106 571 |  |
| Second moment of area (y-y axis) : Iel,yn=26.1 (cm4) | 74 820 | 74 821 |  |
| Second moment of area (y-y axis) : Iel,yn=27.5 (cm4) | 73 533 | 73 534 |  |
| Second moment of area (y-y axis) : Iel,y(steel) (cm4) | 33 742 | 33 743 |  |
| Elastic neutral axis : zel,y(n=6.67) (mm) | 6.83 | 6.82 | 0.15% |
| Elastic neutral axis : zel,y(n=26.1) (mm) | -93.4 | -93.4 |  |
| Elastic neutral axis : zel,y(n=27.5) (mm) | -97.5 | -97.5 |  |
| Elastic neutral axis : zel,y(steel) (mm) | -225 | -225 |  |
| Plastic resistance to bending moment : Mpl,Rd (kN.m) | 846 | 846 |  |

| Case C01 Results: Analysis | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Shear force : V(G1)= (kN) | 50.8 | 50.8 |  |
| Bending moment : M(G1)= (kN.m) | 178 | 178 |  |
| Shear force : V(G2)= (kN) | 29.4 | 29.4 |  |
| Bending moment : M(G2)= (kN.m) | 103 | 103 |  |
| Shear force : V(Q1)= (kN) | 52.5 | 52.5 |  |
| Bending moment : M(Q1)= (kN.m) | 184 | 184 |  |
| Shear force : V(Qc)= (kN) | 15 | 15 |  |
| Bending moment : M(Qc)= (kN.m) | 64.9 | 64.9 |  |

| Case C01 Results: ULS at final stage | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Resistance to bending moment : ΓM= | 0.84 | 0.84 |  |
| Resistance to shear force : ΓV= | 0.23 | 0.23 |  |
| Resistance to MV interaction : ΓMV= | 0.84 | 0.84 |  |
| Degree of shear connection : η= | 0.68 | 0.68 |  |
| Minimum degree of connection : ηmin= | 0.57 | 0.57 |  |
| Longitudinal shear force : τEd= (MPa) | 2.06 | 2.06 |  |
| Min. compression strut inclination angle : θf,min= (°) | 27 | 27 |  |
| Resistance of the compression strut : Γsf = | 0.57 | 0.57 |  |
| Area of reinf. by length unit : Asf/sf = (cm²/m) | 0 | 0 |  |

| Case C01 Results: ULS at construction stage | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Resistance to bending moment : ΓM= | 0.72 | 0.72 |  |
| Resistance to shear force : ΓV= | 0.11 | 0.11 |  |

| Case C01 Results: SLS assuming full shear connection | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Deflection : δ(G1) (mm) | 51.2 | 51.2 |  |
| Deflection : δG2 (mm) | 13.6 | 13.6 |  |
| Deflection : δ(Q1) (mm) | 16.8 | 16.8 |  |
| Deflection : δSHC (mm) | 13.8 | 13.9 | 0.72% |
| Deflection : δSLS (mm) | 95.5 | 95.6 | 0.10% |
| Eigen frequency : fG+0.2Q1 (Hz) | 3.3 | 3.3 |  |

## CASE C01b: Simply supported composite beam

### Description

This test is based on the Construction Métallique review n°2018-2. It deals with a single span composite beam with a composite slab in the construction phase.

It correspond to the construction phase of the Case C01.

### Parameters

All parameters are the same as in Case C01, with the additional parameters:

Lateral restraint of the profile:

No lateral restraints in the span

No restraint by the steel profiled sheeting

Definition of loads:

G1:	self-weight only

live load QC:	surface load 0,5 kN/m2

distributed load :	3 kN/m on the 3 m at the center of the span

Combination of loads:

ULS:	1,35 G1 + 1,5 QC

### Verification of results

| Case C01b Results: ULS at construction stage | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Resistance to bending moment : ΓM= | 0.72 | 0.72 |  |
| Resistance to shear force : ΓV= | 0.11 | 0.11 |  |

## CASE C02: Single span composite beam laterally restrained during its construction phase

### Description

This test is based on the Construction Métallique review review n°2022-2. It deals with a single span composite beam which is laterally restrained by the steel deck during its construction phase.

The reference values are taken from the paper. Because the calculation method of the software is more accurate, some values of the review may not match perfectly.

Figure 2 : Transversal view

### Parameters

All parameters are the same as in Case C01, with the additional parameters:

Lateral restraint of the profile:

No lateral restraints in the span

Restraint by the steel profiled sheeting:

Definition of loads:

G1:	self-weight only

live load QC:	surface load 0,5 kN/m2

distributed load :	3 kN/m on the 3 m at the center of the span

Combination of loads:

ULS:	1,35 G1 + 1,5 QC

### Verification of results

| Case C02 Results: Analysis | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Shear force : V(G1)= (kN) | 50.8 | 50.8 |  |
| Bending moment : M(G1)= (kN.m) | 178 | 178 |  |
| Shear force : V(Qc)= (kN) | 15 | 15 |  |
| Bending moment : M(Qc)= (kN.m) | 64.8 | 64.8 |  |
| Shear force : VEd= (kN) | 91 | 91 |  |
| Bending moment : MEd= (kN.m) | 337 | 337 |  |

| Case C02 Results: ULS during construction stage | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Critical moment to lateral torsional buckling (without flexural bending of deck accounted for) : Mcr (kN.m) | 2 260 | 2 304 | -1.91% |
| Critical moment to lateral torsional buckling (with flexural bending of deck accounted for) : Mcr (kN.m) | 3 237 | 3 299 | -1.88% |
| Resistance to lateral torsional buckling (with flexural bending of deck accounted for) : ΓLT= | 0.77 | 0.77 |  |

## CASE C03: Single span composite beam with partial encasement (construction phase)

### Description

This test is based on the paper in the Construction Métallique review n°2023-3. It deals with a single span composite beam with a composite slab and partial encasement during its construction phase.

The values from the paper are used as references. Because the calculation method of the software is more accurate, some values of the review may not match perfectly.

It correspond to the construction phase of the case C04.

Figure 3 : Transversal view

### Parameters

Definition of the beam:

Definition of the slab:

Composite slab

Total depth:	td=140 mm

Perpendicular ribs

Continuous ribs with un-continuous deck

Steel profiled sheeting:	Cofraplus 60, thickness 1 mm

Prepunched sheeting

Unpropped beam during construction

Definition of the profile:

Hot rolled profile

Profile:	IPE A 500

Steel grade:	S355 M/ML

Definition of the concrete:

Normal concrete

Concrete class:	C25/30

Density:	ρc=2500 kg/m3

Definition of the partial encasement:

Lateral restraint of the profile:

No lateral restraints in the span

No restraint by the steel profiled sheeting

Definition of loads:

G1:	self-weight only

live load QC:	distributed load:	1,86 kN/m on the full length

distributed load :	1,05 kN/m on the 3 m at the centre of the span

Combination of loads:

ULS:	1,35 G1 + 1,5 QC

### Verification of results

| Case C03 Results: Steel profile | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Cross-sectional area : A = (cm2) | 101.3 | 101.1 | 0.15% |
| Shear area : Av = (cm2) | 50.6 | 50.4 | 0.32% |
| Second moment of area (y-y axis) : Iy = (cm4) | 43 014 | 42 933 | 0.19% |
| Second moment of area (z-z axis) : Iz = (cm4) | 1 940 | 1 936 | 0.18% |
| Elastic section modulus about y-y axis : Wel,y = (cm3) | 1 731 | 1 728 | 0.17% |
| Elastic section modulus about z-z axis : Wel,z = (cm3) | 194.0 | 193.6 | 0.18% |
| Plastic section modulus about y-y axis : Wpl,y = (cm3) | 1 950 | 1 946 | 0.18% |
| Plastic section modulus about z-z axis : Wpl,z = (cm3) | 301.7 | 298.3 | 1.15% |
| Torsional moment : It = (cm4) | 64.18 | 64.28 | -0.16% |
| Warping moment : Iw = (cm6) | 1 113 252 | 1 125 230 | -1.06% |
| Mass by length unit : mlin = (kg/ml) | 79.5 | 74.9 | 6.12% |
| Mass : m = (kg) | 994 | 992 | 0.16% |
| Total paint surface : S = (cm2) | 217 643 | 217 643 |  |
| Paint surface (without upper face) : S = (cm2) | 192 643 | 192 643 |  |
| Section factor : M = (m-1) | 172.0 | 172.2 | -0.14% |
| Section factor (without upper face) : M = (m-1) | 152.2 | 152.4 | -0.13% |
| Nominal yield strength : fy = (MPa) | 355 | 355 | 0.00% |
| Axial resistance : Nt,Rd = (kN) | 3 595 | 3 589 | 0.15% |
| Shear resistance : Vpl,Rd = (kN) | 1 036 | 1 033 | 0.32% |
| Plastic resistance to bending : Mpl,Rd = (kN.m) | 692 | 691 | 0.16% |
| Elastic resistance to bending : Mel,Rd = (kN.m) | 614 | 613 | 0.24% |
| Section class for pure bending : | 1 | 1 |  |
| Section class for pure compression : | 4 | 4 |  |
| Shear buckling resistance : hw/tw = | 50.7 | 50.7 |  |
| Shear buckling resistance : 124ε = | 101 | 101 |  |
| Verification of web buckling required? | No | No |  |

| Case C03 Results: Partial encasement | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Notional size (EN 1992-1-1 Annex B) : h0= (mm) | 191 | 191 |  |
| Char. value of the compression strength : fck= (MPa) | 25 | 25 |  |
| Mean value of compression strength : fcm= (MPa) | 33 | 33 |  |
| Mean tensile strength : fctm= (MPa) | 2.56 | 2.56 |  |
| Mean secant modulus of elasticity : Ecm= (MPa) | 31 476 | 31 476 |  |

| Case C03 Results: Slab | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Notional size (EN 1992-1-1 Annex B) : h0= (mm) | 210 | 210 |  |
| Char. value of the compression strength : fck= (MPa) | 25 | 25 |  |
| Mean value of compression strength : fcm= (MPa) | 33 | 33 |  |
| Mean tensile strength : fctm= (MPa) | 2.56 | 2.56 |  |
| Mean secant modulus of elasticity : Ecm= (MPa) | 31 476 | 31 476 |  |
| Sec. moment of area per unit of width : Ipu= (cm4/m) | 60.08 | 60.08 |  |

| Case C03 Results: Section properties at mid-span | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Modular ratio : nenrobage(G1)= | 23.7 | 23.7 |  |
| Modular ratio : nenrobage(Qc)= | 6.67 | 6.67 |  |
| Second moment of area (y-y axis) : Iel,y(n=6.67) (cm4) | 53 002 | 52 921 | 0.15% |
| Second moment of area (y-y axis) : Iel,yn=23.7 (cm4) | 47 200 | 47 128 | -0.15% |
| Second moment of area (y-y axis) : Iel,yn=24.7 (cm4) | 47 083 | 47 005 | -0.17% |
| Elastic neutral axis : zel,y(n=6.67) (mm) | 205 | 205 |  |
| Elastic neutral axis : zel,y(n=23.7) (mm) | 235 | 235 |  |
| Elastic neutral axis : zel,y(n=24.7) (mm) | 236 | 236 |  |
| Plastic resistance to bending moment : Mpl,Rd (kN.m) | 777 | 775 | 0.26% |

| Case C03 Results: Analysis | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Shear force : V(G1)= (kN) | 60.5 | 60.5 |  |
| Bending moment : M(G1)= (kN.m) | 189 | 189 |  |
| Shear force : V(Qc)= (kN) | 13.3 | 13.3 |  |
| Bending moment : M(Qc)= (kN.m) | 45.3 | 45.4 | -0.22% |

| Case C03 results: ULS | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Resistance to bending moment : ΓM= | 0.42 | 0.42 |  |
| Resistance to shear force : ΓV= | 0.10 | 0.10 |  |
| Resistance to MV interaction : ΓMV= | 0.42 | 0.42 |  |
| Critical moment to lateral torsional buckling : Mcr (kN.m) | 903 | 914 | -1.20% |
| Resistance to lateral torsional buckling : ΓLT= | 0.66 | 0.63 | 4.76% |

| Case C03 Results: SLS | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Deflection : δ(G1) (mm) | 31.0 | 31.1 | -0.32% |
| Deflection : δ(Qc) (mm) | 6.5 | 6.5 |  |
| Deflection : δSLS (mm) | 37.5 | 37.6 | -0.27% |

## CASE C04: Single span composite beam with partial encasement (composite phase)

### Description

This test is based on the paper in the Construction Métallique review n°2023-4. It deals with a single span composite beam with a composite slab and partial encasement during its final phase in composite stage.

The values from the paper are used as references. Because the calculation method of the software is more accurate, some values of the review may not match perfectly.

Figure 4 : Transversal view

### Parameters

All parameters are the same as in Case 3, with the following specific parameters:

Definition of the connection:

Studs:	19x100

Two studs in each rib

Unpropped beam during construction

Definition of loads:

G1:	self-weight only

G2:	surface load 1,0 kN/m2

live load Q1:	surface load 2,5 kN/m2

Combination of loads:

ULS:	1,35 (G1+G2) + 1,5 Q1

SLS:	(G1+G2) + Q1

Design parameters:

Standard:	Eurocodes first generation

Design:	According to the class of cross-section

Relative humidity:	50 %

Shrinkage deformation:	325 x 10-6

Age of concrete:	t0=28 d for G2

t = 50 x 365 d

### Verification of results

| Case C04 Results: Steel profile | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Cross-sectional area : A = (cm2) | 101.3 | 101.1 | 0.15% |
| Shear area : Av = (cm2) | 50.6 | 50.4 | 0.32% |
| Second moment of area (y-y axis) : Iy = (cm4) | 43 014 | 42 933 | 0.19% |
| Second moment of area (z-z axis) : Iz = (cm4) | 1 940 | 1 936 | 0.18% |
| Elastic section modulus (y-y axis) : Wel,y = (cm3) | 1 731 | 1 728 | 0.17% |
| Elastic section modulus (z-z axis) : Wel,z = (cm3) | 194.0 | 193.6 | 0.18% |
| Plastic section modulus (y-y axis) : Wpl,y = (cm3) | 1 950 | 1 946 | 0.18% |
| Plastic section modulus (z-z axis) : Wpl,z = (cm3) | 301.7 | 298.3 | 1.15% |
| Torsional constant : It = (cm4) | 64.18 | 64.28 | -0.16% |
| Warping constant : Iw = (cm6) | 1 113 252 | 1 125 230 | -1.06% |
| Mass by length unit : mlin = (kg/ml) | 79.5 | 74.9 | 6.12% |
| Mass : m = (kg) | 994 | 992 | 0.16% |
| Total paint surface : S = (cm2) | 217 643 | 217 643 | 0.00% |
| Paint surface (without upper face) : S = (cm2) | 192 643 | 192 643 | 0.00% |
| Section factor : M = (m-1) | 172.0 | 172.2 | -0.14% |
| Section factor (without upper face) : M = (m-1) | 152.2 | 152.4 | -0.13% |
| Nominal yield strength : fy = (MPa) | 355 | 355 | 0.00% |
| Axial resistance : Nt,Rd = (kN) | 3 595 | 3 589 | 0.15% |
| Shear resistance : Vpl,Rd = (kN) | 1 036 | 1 033 | 0.32% |
| Plastic resistance to bending : Mpl,Rd = (kN.m) | 692 | 691 | 0.16% |
| Elastic resistance to bending : Mel,Rd = (kN.m) | 614 | 613 | 0.24% |
| Section class for pure bending : | 1 | 1 |  |
| Section class for pure compression : | 4 | 4 |  |
| Shear buckling resistance : hw/tw = | 50.7 | 50.7 |  |
| Shear buckling resistance : 124ε = | 101 | 101 |  |
| Verification of web buckling required? | No | No |  |

| Case C04 Results: Partial encasement | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Notional size (EN 1992-1-1 Annex B) : h0= (mm) | 191 | 191 |  |
| Char. value of the compression strength : fck= (MPa) | 25 | 25 |  |
| Mean value of compression strength : fcm= (MPa) | 33 | 33 |  |
| Mean tensile strength : fctm= (MPa) | 2.56 | 2.56 |  |
| Mean secant modulus of elasticity : Ecm= (MPa) | 31 476 | 31 476 |  |

| Case C04 Results: Slab | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Notional size (EN 1992-1-1 Annex B) : h0= (mm) | 164 | 164 |  |
| Char. value of the compression strength : fck= (MPa) | 25 | 25 |  |
| Mean value of compression strength : fcm= (MPa) | 33 | 33 |  |
| Mean tensile strength : fctm= (MPa) | 2.56 | 2.56 |  |
| Mean secant modulus of elasticity : Ecm= (MPa) | 31 476 | 31 476 |  |
| Sec. moment of area per unit of width : Ipu= (cm4/m) | 60.08 | 60.08 |  |
| Shear resistance of studs : PRd= (kN) | 37.4 | 37.4 |  |
| Reduction factor : kt = | 0.50 | 0.50 |  |

| Case C04 Results: Section properties at mid-span | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Effective width of the slab : beff= (m) | 2.50 | 2.50 |  |
| Modular ratio : nslab(G1)= | - | - |  |
| Modular ratio : nencasement(G1)= | 23.7 | 23.7 |  |
| Modular ratio : nslab(G2)= | 26.6 | 26.6 |  |
| Modular ratio : nencasement(G2)= | 23.7 | 23.7 |  |
| Modular ratio : nslab(Q1)= | 6.67 | 6.67 |  |
| Modular ratio : nencasementQ1= | 6.67 | 6.67 |  |
| Modular ratio : nslab(Qc)= | ∞ | ∞ |  |
| Modular ratio : nencasementQc= | 6.67 | 6.67 |  |
| Modular ratio : nslabSHC= | 25.2 | 25.2 |  |
| Modular ratio : nslabSHC= | 24.7 | 24.7 |  |
| Moment of inertia about y-y axis : Iel,y(G1) (cm4) | 47 237 | 47 128 | 0.23% |
| Moment of inertia about y-y axis : Iel,yG2 (cm4) | 99 895 | 99 778 | 0.12% |
| Moment of inertia about y-y axis : Iel,y(Q1) (cm4) | 142 875 | 142 677 | 0.14% |
| Moment of inertia about y-y axis : Iel,y(Qc) (cm4) | 53 094 | 52 921 | 0.33% |
| Moment of inertia about y-y axis : Iel,y(SHC) (cm4) | 101 577 | 101 471 | 0.10 % |
| Elastic neutral axis : zel,y(G1) (mm) | -235 | -235 | 0.00% |
| Elastic neutral axis : zel,y(G2) (mm) | -102.4 | -102 | 0.39% |
| Elastic neutral axis : zel,y(Q1) (mm) | 9.38 | 9.51 | -1.37% |
| Elastic neutral axis : zel,y(Qc) (mm) | -205 | -205 | 0.00% |
| Elastic neutral axis : zel,y(SHC) (mm) | -98.1 | -97.9 | 0.20% |
| Plastic resistance to bending moment : Mpl,Rd (kN.m) | 1 190 | 1 237 | -3.80% |

| Case C04 Results: Analysis | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Shear force : V(G1)= (kN) | 60.5 | 60.5 |  |
| Bending moment : M(G1)= (kN.m) | 189 | 189 |  |
| Shear force : V(G2)= (kN) | 15.6 | 15.6 |  |
| Bending moment : M(G2)= (kN.m) | 48.8 | 48.8 |  |
| Shear force : V(Q1)= (kN) | 39.1 | 39.1 |  |
| Bending moment : M(Q1)= (kN.m) | 122 | 122 |  |
| Shear force : V(Qc)= (kN) | 13.3 | 13.3 |  |
| Bending moment : M(Qc)= (kN.m) | 45.3 | 45.4 | -0.22% |
| Shear force : V(SHC)= (kN) | 0 | 0 |  |
| Bending moment : M(SHC)= (kN.m) | 112.5 | 110 |  |
| Shear force : VEd= (kN) | 161 | 161 |  |
| Bending moment : MEd= (kN.m) | 504 | 504 |  |
| Shear force : VEd,const= (kN) | 102 | 102 |  |
| Bending moment : MEd,const= (kN.m) | 323 | 323 |  |

| Case C04 Results: ULS at final stage | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Resistance to bending moment : ΓM= | 0.42 | 0.42 |  |
| Resistance to shear force : ΓV= | 0.16 | 0.16 |  |
| Resistance to MV interaction : ΓMV= | 0.42 | 0.42 |  |
| Degree of shear connection : η= | 0.78 | 0.78 |  |
| Minimum degree of connection : ηmin= | 0.63 | 0.63 |  |
| Longitudinal shear force : τEd= (MPa) | 2.20 | 2.14 | 2.80% |
| Min. compression strut inclination angle : θf,min= (°) | 27 | 27 |  |
| Resistance of the compression strut : Γsf = | 0.61 | 0.59 | 3.39% |
| Area of reinf. by length unit : Asf/sf = (cm²/m) | 2.12 | 2.05 | 3.41% |

| Case C04 Results: ULS at construction stage | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Resistance to bending moment : ΓM= | 0.42 | 0.42 |  |
| Resistance to shear force : ΓV= | 0.10 | 0.10 |  |
| Resistance to MV interaction : ΓMV= | 0.42 | 0.42 |  |
| Critical moment to lateral torsional buckling : Mcr (kN.m) | 903 | 914 | -1.20% |
| Resistance to lateral torsional buckling : ΓLT= | 0.66 | 0.63 | 4.76% |

| Case C04 Results: SLS assuming full connection | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Deflection : δ(G1) (mm) | 31.1 | 31.1 |  |
| Deflection : δG2 (mm) | 3.79 | 3.79 |  |
| Deflection : δ(Q1) (mm) | 6.62 | 6.57 | 0.76% |
| Deflection : δ(Qc) (mm) | 6.5 | 6.49 | 0.15% |
| Deflection : δSHC (mm) | 10.3 | 10.1 | 1.98% |
| Deflection : δSLS (mm) | 51.8 | 51.5 | 0.58% |
| Eigen frequency : fG+0.2Q1 (Hz) | 4.6 | 4.72 | -2.54% |

# Tests cases – Steel beams

## CASE S01: Steel beam with 2 cantilever spans

### Description

This test deals with a steel beam with both left and right cantilevers.

Figure 5 : Transversal view

### Parameters

Definition of the beam:

Definition of the profile:

Hot rolled profile

Profile:	IPE 450

Steel grade:	S355 M/ML

Definition of the slab:

Lateral restraint of the profile:

One lateral restraints at mid-span of the central span, for both flanges

Definition of loads:

G1:	self-weight only

live load Q1:	left cantilever span: 	distributed load from 0 (left end) to 3 kN/m2 (right end)

central span: 	distributed load from 0 (left end) to 10 kN/m2 (right end)

right cantilever span: 	distributed load from 0 (left end) to 3 kN/m2 (right end)

### Verification of results

| Case S01 Results: steel profile | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Cross-sectional area : A = (cm2) | 98.8 | 98.8 |  |
| Shear area : Av = (cm2) | 50.8 | 50.8 |  |
| Second moment of area (y-y axis) : Iy = (cm4) | 33 740 | 33 743 | 0.01% |
| Second moment of area (z-z axis) : Iz = (cm4) | 1 676 | 1 672 | 0.24% |
| Elastic section modulus (y-y axis) : Wel,y = (cm3) | 1 500 | 1 500 |  |
| Elastic section modulus (z-z axis) : Wel,z = (cm3) | 176 | 176 |  |
| Plastic section modulus (y-y axis) : Wpl,y = (cm3) | 1 702 | 1 702 |  |
| Plastic section modulus (z-z axis) : Wpl,z = (cm3) | 276 | 272.8 | 1.17% |
| Torsional constant : It = (cm4) | 66.66 | 66.75 | 0.13% |
| Warping constant : Iw = (cm6) | 780 739 | 791 005 | 1.30% |
| Mass by length unit : mlin = (kg/ml) | 77.7 | 77.6 | 0.13% |
| Mass : m = (kg) | 1 243 | 1 241 | 0.16% |
| Total paint surface : S = (cm2) | 256 824 | 256 824 |  |
| Paint surface (without upper face) : S = (cm2) | 226 424 | 226 424 |  |
| Section factor : M = (m-1) | 162.4 | 162.4 |  |
| Section factor (without upper face) : M = (m-1) | 143.2 | 143.2 |  |
| Nominal yield strenght : fy = (MPa) | 355 | 355 |  |
| Axial resistance : Nt,Rd = (kN) | 3 508 | 3 508 |  |
| Shear resistance : Vpl,Rd = (kN) | 1 402 | 1 402 |  |
| Plastic moment resistant : Mpl,Rd = (kN.m) | 604 | 604 |  |
| Elastic moment resistant : Mel,Rd = (kN.m) | 532 | 532 |  |
| Section class for bending : | 1 | 1 |  |
| Section class for compression : | 4 | 4 |  |
| Shear buckling resistance : hw/tw = | 40.3 | 40.3 |  |
| Shear buckling resistance : 72ε/η = | 48.8 | 48.8 |  |
| Verification of web buckling required? | No | No |  |

| Case S01 Results: Analysis | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Shear force : V(G1)= (kN) | 34.4 | 34.2 | 0.58% |
| Bending moment : M(G1)= (kN.m) | 55.1/-31 | 54.7/-30.8 | 0.73%/0.65% |
| Shear force : V(Q1#1)= (kN) | 13.5 | 13.5 |  |
| Bending moment : M(Q1#1)= (kN.m) | 14.7/-19.5 | 14.7/-19.5 |  |
| Shear force : V(Q1#2)= (kN) | 11.7 | 11.7 |  |
| Bending moment : M(Q1#2)= (kN.m) | 25.2 | 25.2 |  |
| Shear force : V(Q1#3)= (kN) | 12 | 12 |  |
| Bending moment : M(Q1#3)= (kN.m) | -19.5 | -19.5 |  |
| Shear force : VEd,ULS#1= (kN) | 66.7 | 66.4 | 0.45% |
| Bending moment : MEd,ULS#1= (kN.m) | 96.2/-71.1 | 95.7/-70.8 | 0.52%/0.42% |
| Shear force : VEd,ULS#2= (kN) | 64.0 | 63.7 | 0.47% |
| Bending moment : MEd,ULS#2= (kN.m) | 112/-41.8 | 111/-41.6 | 0.90%/0.48% |
| Shear force : VEd,ULS#3= (kN) | 49.2 | 48.9 | 0.61% |
| Bending moment : MEd,SLS#3= (kN.m) | 59.0/-71.1 | 58.5/-70.8 | 0.85%/0.42% |

| Case S01 Results: ULS | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Resistance to bending moment : ΓM,ULS#1= | 0.16 | 0.16 | 0.00% |
| Resistance to shear force : ΓV,ULS#1= | 0.06 | 0.06 | 0.00% |
| Resistance to MV interaction : ΓMV,ULS#1= | 0.16 | 0.16 | 0.00% |
| Critical moment to lateral torsional buckling : Mcr,max,ULS#1 (kN.m) | 521.97 | 523.4 | -0.27% |
| Resistance to lateral torsional buckling : ΓLT,ULS#1= | 0.29 | 0.29 | 0.00% |
| Resistance to bending moment : ΓM,ULS#2= | 0.19 | 0.18 | 5.56% |
| Resistance to shear force : ΓV,ULS#2= | 0.06 | 0.06 | 0.00% |
| Resistance to MV interaction : ΓMV,ULS#2= | 0.19 | 0.18 | 5.56% |
| Critical moment to lateral torsional buckling : Mcr,max,ULS#2 (kN.m) | 546.2 | 543.3 | 0.53% |
| Resistance to lateral torsional buckling : ΓLT,ULS#2= | 0.33 | 0.33 | 0.00% |
| Resistance to bending moment : ΓM,ULS#3= | 0.12 | 0.12 | 0.00% |
| Resistance to shear force : ΓV,ULS#3= | 0.05 | 0.05 | 0.00% |
| Resistance to MV interaction : ΓMV,ULS#3= | 0.12 | 0.12 | 0.00% |
| Critical moment to lateral torsional buckling : Mcr,max,ULS#3 (kN.m) | 496.6 | 497.1 | -0.10% |
| Resistance to lateral torsional buckling : ΓLT,ULS#3= | 0.22 | 0.22 | 0.00% |

| Case S01 Results: sls | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Deflection : δ(G1) (mm) | 7.19 | 7.14 |  |
| Deflection : δ(Q1#1) (mm) | 1.83 | 1.83 | 0.00% |
| Deflection : δ(Q1#2) (mm) | 3.68 | 3.68 | 0.00% |
| Deflection : δ(Q1#3) (mm) | 3.50 | 3.50 |  |
| Deflection : δ(SLS#1) (mm) | 9.01 | 8.96 |  |
| Deflection : δ(SLS#2) (mm) | 10.9 | 10.8 |  |
| Deflection : δ(SLS#3) (mm) | 5.34 | 5.30 |  |

## CASE S02: Steel beam with welded profile

### Description

This test deals with a steel beam with welded profile (double symmetric).

Figure 6 : Transversal view

### Parameters

Definition of the beam:

Single span

Span length:	15 m

Definition of the profile:

Welded profile

Upper flange:	300x20

Web:	800x12

Lower flange:	300x200

Steel grade:	S355 JR

Definition of loads:

G1:	distributed load:	12 kN/m

live load Q1:	distributed load:	20 kN/m

live load Q2:	concentrated load:	15 kN at mid span

Lateral restraints:

2 lateral restraints of both flange at 1/3 and 2/3 of the span length

Partial factors and national factors:

γG=1,4

γQ=1,6

γM0=1,05

γM1=1,1

γM2=1,25

ψ0=0,7

η=1,2

Combination of loads:

ULS-01:	γG G+γQ Q1+γQ ψ0 Q2

ULS-02:	γG G+γQ ψ0 Q1+γQ  Q2

SLS:	G+Q1+Q2

Design parameters:

Standard:	Eurocodes first generation

### Verification of results

| Case S02 Results: steel profile | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Cross-sectional area : A = (cm2) | 216 |  |  |
| Shear area : Av = (cm2) | 115,2 |  |  |
| Second moment of area (y-y axis) : Iy = (cm4) | 252 960 |  |  |
| Second moment of area (z-z axis) : Iz = (cm4) | 9012 |  |  |
| Elastic section modulus (y-y axis) : Wel,y = (cm3) | 6023 |  |  |
| Torsional constant : It = (cm4) |  |  |  |
| Warping constant : Iw = (cm6) |  |  |  |
| Mass by length unit : mlin = (kg/ml) | 170 |  |  |
| Mass : m = (kg) | 2543 |  |  |
| Total paint surface : S = (cm2) | 42,84 |  |  |
| Paint surface (without upper face) : S = (cm2) | 38,34 |  |  |
| Section factor : M = (m-1) |  |  |  |
| Section factor (without upper face) : M = (m-1) |  |  |  |
| Nominal yield strength flanges : fy = (MPa) | 345 |  |  |
| Nominal yield strength web : fy = (MPa) | 355 |  |  |
| Axial resistance : Nt,Rd = (kN) |  |  |  |
| Shear resistance : Vpl,Rd = (kN) |  |  |  |
| Plastic bending resistance (y-y axis) : Mpl,Rd = (kNm) | 2266 |  |  |
| Elastic bending resistance (y-y axis) : Mel,Rd = (kNm) | 1979 |  |  |
| Section class for bending : | 2 |  |  |
| Section class for compression : |  |  |  |
| Shear buckling resistance : hw/tw = | 66,67 |  |  |
| Shear buckling resistance : 72ε/η = | 48,82 |  |  |
| Verification of web buckling required? | Yes |  |  |

| Case S02 Results: Analysis | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Shear force : V(G1)= (kN) | 90 |  |  |
| Bending moment : M(G1)= (kN.m) | 337,5 |  |  |
| Shear force : V(Q1)= (kN) | 150 |  |  |
| Bending moment : M(Q1)= (kN.m) | 562.5 |  |  |
| Shear force : V(Q2)= (kN) | 7,5 |  |  |
| Bending moment : M(Q2)= (kN.m) | 56,3 |  |  |
| Shear force : VEd,ULS-1= (kN) | 374,4 |  |  |
| Bending moment : MEd,ULS-1= (kN.m) | 1435,5 |  |  |
| Shear force : VEd,ULS-2= (kN) | 306 |  |  |
| Bending moment : MEd,ULS-2= (kN.m) | 1192,5 |  |  |

| Case S02 Results: ULS | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Resistance to bending moment : ΓM | 0,634 |  |  |
| Resistance to shear force : ΓV | 0,200 |  |  |
| Resistance to shear buckling : ΓVb | 0,239 |  |  |
| Resistance to MV interaction : ΓMV | - |  |  |
| Critical moment to lateral torsional buckling : Mcr,max,ULS-1 (kN.m) | 4163 |  |  |
| Resistance to lateral torsional buckling : ΓLT= | 1,09 |  |  |
| Minimum design size of welds: aw (mm) | 0,75 |  |  |

| Case S02 Results: sls | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Deflection : δ(G) (mm) | 14,9 |  |  |
| Deflection : δ(Q1) (mm) | 24,8 |  |  |
| Deflection : δ(Q2) (mm) | 1,99 |  |  |
| Deflection : δ(SLS) (mm) | 41,1 |  |  |
| Eigen frequency : f(G+0,2 Q1) (Hz) | 3,98 |  |  |
| Modal mass: (kg) | 12 232 |  |  |

## CASE S02b: Steel beam with welded profile

### Description

This test deals with a steel beam with welded profile (double symmetric). An elastic design is triggered by the class 3 section

Figure 7 : Transversal view

### Parameters

All parameters are the same as in Case S02, with the following modifications:

Definition of the profile:

Welded profile

Upper flange:	300x20

Web:	800x12

Lower flange:	300x200

Steel grade:	S460 JR

Definition of loads:

G1:	distributed load:	12 kN/m

live load Q1:	distributed load:	20 kN/m

live load Q2:	concentrated load:	30 kN at mid span

### Verification of results

| Case S02b Results: steel profile | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Cross-sectional area : A = (cm2) | 216 |  |  |
| Shear area : Av = (cm2) | 115,2 |  |  |
| Second moment of area (y-y axis) : Iy = (cm4) | 252 960 |  |  |
| Second moment of area (z-z axis) : Iz = (cm4) | 9012 |  |  |
| Elastic section modulus (y-y axis) : Wel,y = (cm3) | 6023 |  |  |
| Torsional constant : It = (cm4) |  |  |  |
| Warping constant : Iw = (cm6) |  |  |  |
| Mass by length unit : mlin = (kg/ml) | 170 |  |  |
| Mass : m = (kg) | 2543 |  |  |
| Total paint surface : S = (cm2) | 42,84 |  |  |
| Paint surface (without upper face) : S = (cm2) | 38,34 |  |  |
| Section factor : M = (m-1) |  |  |  |
| Section factor (without upper face) : M = (m-1) |  |  |  |
| Nominal yield strength flanges : fy = (MPa) | 440 |  |  |
| Nominal yield strength web : fy = (MPa) | 460 |  |  |
| Axial resistance : Nt,Rd = (kN) |  |  |  |
| Shear resistance : Vpl,Rd = (kN) |  |  |  |
| Plastic bending resistance (y-y axis) : Mpl,Rd = (kNm) | 2524 |  |  |
| Elastic bending resistance (y-y axis) : Mel,Rd = (kNm) | 2903 |  |  |
| Section class for bending : | 3 |  |  |
| Section class for compression : |  |  |  |
| Shear buckling resistance : hw/tw = | 66,67 |  |  |
| Shear buckling resistance : 72ε/η = | 42,89 |  |  |
| Verification of web buckling required? | Yes |  |  |

| Case S02b Results: Analysis | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Shear force : V(G1)= (kN) | 90 |  |  |
| Bending moment : M(G1)= (kN.m) | 337,5 |  |  |
| Shear force : V(Q1)= (kN) | 150 |  |  |
| Bending moment : M(Q1)= (kN.m) | 562.5 |  |  |
| Shear force : V(Q2)= (kN) | 15 |  |  |
| Bending moment : M(Q2)= (kN.m) | 112,5 |  |  |
| Shear force : VEd,ULS-1= (kN) | 382,8 |  |  |
| Bending moment : MEd,ULS-1= (kN.m) | 1498,5 |  |  |
| Shear force : VEd,ULS-2= (kN) | 318 |  |  |
| Bending moment : MEd,ULS-2= (kN.m) | 1282,5 |  |  |

| Case S01 Results: ULS | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Resistance to bending moment : ΓM | 0,594 |  |  |
| Resistance to shear force : ΓV | 0,158 |  |  |
| Resistance to shear buckling : ΓVb | 0,215 |  |  |
| Resistance to MV interaction : ΓMV | - |  |  |
| Critical moment to lateral torsional buckling : Mcr,max,ULS-1 (kN.m) | 4189 |  |  |
| Resistance to lateral torsional buckling : ΓLT= | 1,07 |  |  |
| Minimum design size of welds: aw (mm) | 0,75 |  |  |

| Case S02 Results: sls | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Deflection : δ(G) (mm) | 14,9 |  |  |
| Deflection : δ(Q1) (mm) | 24,8 |  |  |
| Deflection : δ(Q2) (mm) | 3,97 |  |  |
| Deflection : δ(SLS) (mm) | 42,5 |  |  |
| Eigen frequency : f(G+0,2 Q1) (Hz) | 3,98 |  |  |
| Modal mass: (kg) | 12 232 |  |  |

## CASE S03: Steel beam with welded profile and MV interaction

### Description

This test deals with a steel beam with welded profile and a MV interaction.

Figure 8 : Transversal view

### Parameters

Definition of the beam:

Single span with a cantilever span on the left

Span length:	9 m

Cantilever length:	2,5 m

Definition of the profile:

Welded profile

Upper flange:	150x18

Web:	280x6

Lower flange:	150x22

Steel grade:	S275 JR

Definition of loads:

live load Q1:	distributed load on the cantilever span:	75 kN/m

Lateral restraints:

2 lateral restraints of both flange at 1/3 and 2/3 of the span length

Partial factors and national factors:

γM0=1,0

γM1=1,0

γM2=1,25

η=1,0

Combination of loads:

ULS-01:	Q1

SLS:	Q1

Design parameters:

Standard:	Eurocodes first generation

### Verification of results

| Case S03 Results: steel profile | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Cross-sectional area : A = (cm2) | 76,80 | 76,80 |  |
| Shear area : Av = (cm2) | 16,8 | 16,8 |  |
| Second moment of area (y-y axis) : Iy = (cm4) | 14 517 | 14 517 |  |
| Location of the elastic neutral axis: zy,ENA = (mm) | 170,5 | 170,5 |  |
| Second moment of area (z-z axis) : Iz = (cm4) | 1 125 | 1 126 |  |
| Torsional constant : It = (cm4) |  |  |  |
| Warping constant : Iw = (cm6) |  |  |  |
| Mass by length unit : mlin = (kg/ml) | 60 | 60 |  |
| Mass : m = (kg) | 693 | 693 |  |
| Total paint surface : S = (cm2) | 14,12 | 14,12 |  |
| Paint surface (without upper face) : S = (cm2) | 12,40 | 12,40 |  |
| Section factor : M = (m-1) |  |  |  |
| Section factor (without upper face) : M = (m-1) |  |  |  |
| Nominal yield strength flanges : fy = (MPa) | 275 | 275 |  |
| Nominal yield strength web : fy = (MPa) | 275 | 275 |  |
| Axial resistance : Nt,Rd = (kN) | 2 112 | 2 112 |  |
| Shear resistance : Vpl,Rd = (kN) | 267,0 | 267 |  |
| Plastic bending resistance (y-y axis) : Mpl,Rd = (kNm) | 276 | 276 |  |
| Location of the plastic neutral axis: zy,PNA = (mm) | 208 | 208 |  |
| Elastic bending resistance (y-y axis) : Mel,Rd = (kNm) | 234 | 234 |  |
| Section class for bending : | 1 | 1 |  |
| Section class for compression : |  |  |  |
| Shear buckling resistance : hw/tw = | 46,67 | 46,7 |  |
| Shear buckling resistance : 72ε/η = | 66,56 | 66,56 |  |
| Verification of web buckling required? | No | No |  |

| Case S03 Results: Analysis | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Shear force : V(Q1)= (kN) | 187,5 | 188 |  |
| Bending moment : M(Q1)= (kN.m) | -234,4 | -234 |  |

| Case S03 Results: ULS | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Resistance to bending moment : ΓM | 0,850 | 0,850 |  |
| Resistance to shear force : ΓV | 0,703 | 0,703 |  |
| Resistance to shear buckling : ΓVb | - |  |  |
| Resistance to MV interaction : ΓMV | 0,869 | 0,869 |  |
| Critical moment to lateral torsional buckling : Mcr,max,ULS-1 (kN.m) |  |  |  |
| Resistance to lateral torsional buckling : ΓLT= |  |  |  |
| Minimum design size of welds: aw (mm) |  |  |  |

| Case S03 Results: sls | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Deflection at the end of the cantilever : δ(Q1) (mm) | 69,7 | 69,7 |  |
| Deflection at mid span : δ(Q1) (mm) | -39,9 | -39,9 |  |

## CASE S03b: Steel beam with welded profile and MV interaction

### Description

This test deals with a steel beam with welded profile and a MV interaction.

Figure 8 : Transversal view

### Parameters

The case S03b is identical to Case S03, except that the cantilever is defined on the right end of the beam.

### Verification of results

| Case S03b Results: Analysis | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Shear force : V(Q1)= (kN) | 187,5 | 188 |  |
| Bending moment : M(Q1)= (kN.m) | -234,4 | -234 |  |

| Case S03b Results: ULS | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Resistance to bending moment : ΓM | 0,850 | 0,850 |  |
| Resistance to shear force : ΓV | 0,703 | 0,703 |  |
| Resistance to shear buckling : ΓVb | - |  |  |
| Resistance to MV interaction : ΓMV | 0,869 | 0,869 |  |
| Critical moment to lateral torsional buckling : Mcr,max,ULS-1 (kN.m) | -277,3 | 279,4 |  |
| Resistance to lateral torsional buckling : ΓLT= |  |  |  |
| Minimum design size of welds: aw (mm) |  |  |  |

| Case S03b Results: sls | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Deflection at the end of the cantilever : δ(Q1) (mm) | 69,7 | 69,7 |  |
| Deflection at mid span : δ(Q1) (mm) | -39,9 | -39,9 |  |

## CASE 6: Simply supported composite beam with welded steel section

### Description

This test deals with a simple welded steel beam. Because the calculation method of the software is more accurate, some values of the review may not match perfectly.

Figure 9 : Transversal view

### Parameters

Definition of the beam:

Span length: 	10 m

Intermediate beam

Spacing of adjacent beams: 2 m

Figure 10 : Definition of the slab

Figure 11 : Definition of the steel profile

Figure 12 : Definition of the connection

Figure 13 : Definition of the lateral restraints

Figure 14 : Propping of the beam

Figure 15 : Restraint of the beam by steel deck (construction phase only)

Figure 16 : Definition of the load cases – Permanent loads G1

Figure 17 : Definition of the load cases – Additional permanent loads G2

Figure 18 : Definition of the load cases – Live load 1 Q1

Figure 19 : Definition of the load cases – Live load 2 Q2

Figure 20 : Definition of the load cases – Construction phase live load QC

Figure 21 : Definition of national factors

Figure 22 : Combinations – ULS and SLS

Figure 23 : Combinations – FLS and construction stage

Figure 24 : Beam calculation options

Figure 25 : Floor vibrations parameters

### Results

| Résultats | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Cross-sectional area : A = (cm2) | 43.4 | 43.4 | 0.00% |
| Shear area : Av = (cm2) | 20.1 | 20.1 | 0.00% |
| Moment of inertia about y-y axis : Iy = (cm4) | 5 921 | 5 921 | 0.00% |
| Moment of inertia about z-z axis : Iz = (cm4) | 388 | 388 | 0.00% |
| Elastic section modulus about y-y axis : Wel,y = (cm3) | 332.6 | 332.6 | 0.00% |
| Elastic section modulus about z-z axis : Wel,z = (cm3) | 51.8 | 51.8 | 0.00% |
| Plastic section modulus about y-y axis : Wpl,y = (cm3) | 455.0 | 455.0 | 0.00% |
| Plastic section modulus about z-z axis : Wpl,z = (cm3) | 84.2 | 85.4 | -0.24% |
| Torsional moment : It = (cm4) | 10.29 | 10.34 | -0.48% |
| Warping moment : Iw = (cm6) | 57 128 | 57 098 | -0.05% |
| Linear mass : mlin = (kg/ml) | 34.0 | 34.0 | 0.00% |
| Mass : m = (kg) | 340.4 | 340.4 | 0.00% |
| Total paint surface : S = (cm2) | 112 580 | 112 580 | 0.00% |
| Paint surface (without upper face) : S = (cm2) | 100 580 | 100 580 | 0.00% |
| Massiveness : M = (m-1) | 259.6 | 259.6 | 0.00% |
| Massiveness (without upper face) : M = (m-1) | 231.9 | 231.9 | 0.00% |
| Nominal yield strenght : fy,s = (MPa) | 420 | 420 | 0.00% |
| Nominal yield strenght : fy,w = (MPa) | 420 | 420 | 0.00% |
| Nominal yield strenght : fy,i = (MPa) | 420 | 420 | 0.00% |
| Young’s modulus : E = (MPa) | 210 000 | 210 000 | 0.00% |
| Density : ρ = (kg/m3) | 7 850 | 7 850 | 0.00% |
| Tensile resistance : Nt,Rd = (kN) | 1 821 | 1 821 | 0.00% |
| Shear resistance : Vpl,Rd = (kN) | 488 | 488 | 0.00% |
| Plastic moment resistant : Mpl,Rd = (kN.m) | 191 | 191 | 0.00% |
| Elastic moment resistant : Mel,Rd = (kN.m) | 140 | 140 | 0.00% |
| Section class for bending : | 3 | 3 | 0.00% |
| Section class for compression : | 4 | 4 | 0.00% |
| Shear buckling resistance : hw/tw = | 39.9 | 39.9 | 0.00% |
| Shear buckling resistance : 72ε/η = | 53.9 | 53.9 | 0.00% |
| Verification of web buckling necessary ? | No | No | Ok |

Table 1: Steel profile

| Résultats | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Notional size (EN 1992-1-1 Annex B) : h0= (mm) | 124 | 124 | 0.00% |
| Char. value of the compression strenght : fck= (MPa) | 20 | 20 | 0.00% |
| Mean value of compression strenght : fcm= (MPa) | 28 | 28 | 0.00% |
| Mean tensile strenght : fctm= (MPa) | 2.21 | 2.21 | 0.00% |
| Mean secant modulus of elasticity : Ecm= (MPa) | 29 962 | 29 962 | 0.00% |
| Shear resistance : PRd= (kN) | 64.8 | 64.8 | 0.00% |

Table 2: Slab

| Résultats | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Effective width of the slab : beff= (m) | 2.0 | 2.0 | 0.00% |
| Modular ratio : nslab(G1)= | ∞ | ∞ | 0.00% |
| Modular ratio : nslab(G2)= | 30.8 | 30.8 | 0.00% |
| Modular ratio : nslab(Q1)= | 7.01 | 7.01 | 0.00% |
| Modular ratio : nslab(Qc)= | ∞ | ∞ | 0.00% |
| Modular ratio : nslabSHC= | 29.1 | 29.1 | 0.00% |
| Moment of inertia about y-y axis : Iel,y(G1) (cm4) | 5 921 | 5 921 | 0.00% |
| Moment of inertia about y-y axis : Iel,yG2 (cm4) | 22 644 | 22 581 | 0.28% |
| Moment of inertia about y-y axis : Iel,y(Q1) (cm4) | 31 404 | 31 404 | 0.00% |
| Moment of inertia about y-y axis : Iel,y(Qc) (cm4) | 5 921 | 5 921 | 0.00% |
| Moment of inertia about y-y axis : Iel,y(SHC) (cm4) | 23 006 | 22 952 | 0.24% |
| Elastic neutral axis : zel,y(G1) (mm) | -178 | -178 | 0.00% |
| Elastic neutral axis : zel,y(G2) (mm) | -25.1 | -26.8 | -6.34% |
| Elastic neutral axis : zel,y(Q1) (mm) | 38.8 | 38.8 | 0.00% |
| Elastic neutral axis : zel,y(Qc) (mm) | -178 | -178 | 0.00% |
| Elastic neutral axis : zel,y(SHC) (mm) | -22.1 | -23.7 | -6.75% |
| Plastic resistance to bending moment : Mpl,Rd (kN.m) | 470 | 470 | 0.00% |
| Including reduction factor : β | 0.975 | 0.975 | 0.00% |

Table 3: Section properties at mid-span

| Résultats | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Shear force : V(G1)= (kN) | 31.1 | 31.1 | 0.00% |
| Bending moment : M(G1)= (kN.m) | 77.7 | 77.7 | 0.00% |
| Shear force : V(G2)= (kN) | 20 | 20 | 0.00% |
| Bending moment : M(G2)= (kN.m) | 50 | 50 | 0.00% |
| Shear force : V(Q1)= (kN) | 0.5 | 0.5 | 0.00% |
| Bending moment : M(Q1)= (kN.m) | 2.5 | 2.5 | 0.00% |
| Shear force : V(Qc)= (kN) | 2.4 | 2.4 | 0.00% |
| Bending moment : M(Qc)= (kN.m) | 9.6 | 9.6 | 0.00% |
| Shear force : V(SHC)= (kN) | 0 | 0 | 0.00% |
| Bending moment : M(SHC)= (kN.m) | 49 | 45.7 | 7.22% |
| Shear force : VEd= (kN) | 69.7 | 69.7 | 0.00% |
| Bending moment : MEd= (kN.m) | 176.2 | 176.1 | 0.06% |
| Shear force : VEd,const= (kN) | 45.6 | 46.5 | 0.00% |
| Bending moment : MEd,const= (kN.m) | 117 | 117 | 0.00% |

Table 4: Analysis

| Résultats | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Class of the composite section : | 3 | 3 | 0.00% |
| Resistance to bending moment : ΓM= | 0.91 | 0.90 | 1.11% |
| Resistance to shear force : ΓV= | 0.14 | 0.14 | 0.00% |
| Resistance to MV interaction : ΓMV= | 0.52 | 0.51 | 1.96% |
| Resistance of connection : Γconnex= | 0.22 | 0.23 | 4.35% |
| Web to upper flange welds design value : aws,calcul= (mm) | 0.36 | 0.36 | 0.00% |
| Web to lower flange welds design value : awi,calcul= (mm) | 0.39 | 0.39 | 0.00% |
| Longitudinal shear force (a-a) : τEd,a-a= (MPa) |  | 1.55 |  |
| Min. compression strut inclination angle (a-a) : θf,min,a-a= (°) |  | 27.0 |  |
| Resistance of the compression strut (a-a) : Γsf,a-a = |  | 0.52 |  |
| Area of reinf. by length unit (a-a) : Asf/sfa-a = (cm²/m) |  | 2.00 |  |
| Longitudinal shear force (d-d) : τEd,d-d= (MPa) |  | 1.11 |  |
| Min. compression strut inclination angle (d-d) : θf,min,d-d= (°) |  | 27.0 |  |
| Resistance of the compression strut (d-d) : Γsf,d-d = |  | 0.37 |  |
| Area of reinf. by length unit (d-d) : Asf/sfd-d = (cm²/m) |  | 4.00 |  |

Table 5: ULS at final stage

| Résultats | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Class of the steel section : |  | 3 |  |
| Resistance to bending moment : ΓM= |  | 0.75 |  |
| Resistance to shear force : ΓV= |  | 0.09 |  |
| Resistance to MV interaction : ΓMV= |  | 0.09 |  |
| Critical moment to lateral torsional buckling : Mcr (kN.m) |  | 146.5 |  |
| Resistance to lateral torsional buckling : ΓLT= |  | 1.75 |  |
| Web to upper flange welds design value : aws= (mm) |  | 0.20 |  |
| Web to lower flange welds design value : awi= (mm) |  | 0.30 |  |

Table 6: ULS at construction stage

| Résultats | Reference | ABCPMX | Dev. |
| --- | --- | --- | --- |
| Deflection : δ(G1) (mm) |  |  |  |
| Deflection : δG2 (mm) |  |  |  |
| Deflection : δ(Q1) (mm) |  |  |  |
| Deflection : δ(Qc) (mm) |  |  |  |
| Deflection : δSHC (mm) |  |  |  |
| Deflection : δSLS (mm) |  |  |  |
| Eigen frequency : fG+0.2Q1 (Hz) |  |  |  |

Table 7: SLS assuming full connection